- Download and unzip the Realtek update tool via the CMI Knowledge Base

- Connect your computer to a power supply

- Connect the Micro USB Ethernet Adapter to your computer using a USB-C to micro-USB adapter

- Connect the Ethernet Adapter’s USB-A connector to your computer or USB power outlet.

- Double click on “RtPGtoolUI.exe” to open the tool

- Click the “Dump” button [1]

- Make sure the “Fix MacAddress” box is checked [2]

- Change the last digit of EXT_PATCH to “4” [3]

- Click on “Program” [4]

- Wait for the result to be “Pass” [5]

- Power cycle the adapter by removing both connectors, and then reconnecting them

- Click on “Dump” and verify that EXT_PATCH is flashed to 00000004